| Frequency band (GHz) | Allocation RR Region 1 | National use | Notes |
| --- | --- | --- | --- |
| 10 - 10.4 | FIXED MOBILE RADIOLOCATION Amateur EARTH EXPLORATION SATELLITE (active) 5.474A 5.474B 5.474C 5.479 5.474D | FIXED, FWA (10.15-10.3 paired with 10.5-10.65) RADIOLOCATION Amateur SRD- Short Range Devices (Tank Level Prob Radar 8.5-10.6 GHz) (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-05 Amateur Reg CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC (06)08 |
| 10.4 – 10.45 | FIXED MOBILE RADIOLOCATION Amateur | FIXED, FWA (10.15-10.3 paired with 10.5-10.65) RADIOLOCATION Amateur SRD- Short Range Devices (Tank Level Prob Radar 8.5-10.6 GHz) (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-05 Amateur Reg CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 10.45 – 10.5 | RADIOLOCATION Amateur Amateur-Satellite | RADIOLOCATION Amateur Amateur-Satellite SRD- Short Range Devices (Tank Level Prob Radar 8.5-10.6 Hz) (Radiodeterm Applic. 30 MHz-12,4 GHz) | Amateur Reg CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 10.5 - 10.55 | FIXED MOBILE Radiolocation | FIXED, FWA (10.5-10.65 paired with 10.15-10.3) Radiolocation SRD- Short Range Devices (Tank Level Prob Radar 8.5-10.6 GHz) (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-05 CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 10.55 - 10.6 | FIXED MOBILE except aeronautical mobile Radiolocation | FIXED (10.5-10.65 paired with 10.15-10.3) Radiolocation SRD- Short Range Devices (Tank Level Prob Radar 8.5-10.6 GHz) (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-05 CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments ECC/DEC (06)08 |
| 10.6 - 10.68 | EARTH EXPLORATION SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) Radiolocation 5.482A | FIXED (10.5-10.65 paired with 10.15-10.3) SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-05 ECC/DEC/(10)01 CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 10.68 - 10.7 | EARTH EXPLORATION SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340, 5.483 | SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 10.7 – 10.95 | FIXED FIXED-SATELLITE (space to earth) 5.441, (earth to space) 5.484 MOBILE except aeronautical mobile | FIXED-SATELLITE (space to earth) - VSAT down link - SNG down link - HEST downlink - ESV downlink - AES downlink - ESIM downlink SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-06 ERC DEC (00)08 RR Appendix 30B ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 ECC/DEC(19)04 ECC/DEC (03)04 ECC/DEC/(06)03 ECC/DEC(05(10) ECC/DEC/(05)11 ERC/DEC/(99)26 CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 10.95 – 11.2 | FIXED FIXED-SATELLITE (space to earth) 5.484A 5.484b (earth to space) 5.484 MOBILE except aeronautical mobile | FIXED-SATELLITE (space to earth) - VSAT down link - SNG down link - HEST downlink - ESV downlink - AES downlink - ESIM downlink SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-06 ERC DEC (00)08 RR Appendix 30B ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 ECC/DEC(19)04 ECC/DEC (03)04 ECC/DEC/(06)03 ECC/DEC(05(10) ECC/DEC/(05)11 ERC/DEC/(99)26 ECC/DEC (06)08 CEPT T/R 70-03 |
| 11.2 – 11.45 | FIXED FIXED-SATELLITE (space to earth) 5.441, (earth to space) 5.484 MOBILE except aeronautical mobile | FIXED-SATELLITE (space to earth) - VSAT down link - SNG down link - HEST downlink - ESV downlink - AES downlink - ESIM downlink SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-06 ERC DEC (00)08 RR Appendix 30B ECC/DEC (03)04 ECC/DEC/(06)03 ECC/DEC(05(10) ECC/DEC/(05)11 ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 ECC/DEC(19)04 ERC/DEC/(99)26 CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 11.45 – 11.7 | FIXED FIXED-SATELLITE (space to earth) 5.484A 5.484b (earth to space) 5.484 MOBILE except aeronautical mobile | FIXED-SATELLITE (space to earth) - VSAT down link - SNG down link - HEST downlink - ESV downlink - AES downlink - ESIM downlink SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | CEPT T/R 12-06 ERC DEC (00)08 RR Appendix 30B ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 ECC/DEC(19)04 ECC/DEC (03)04 ECC/DEC/(06)03 ECC/DEC(05(10) ECC/DEC/(05)11 ERC/DEC/(99)26 CEPT T/R 70-03 ECC/DEC (06)08 Commison Dec 2006/771/EC + amendments |
| 11.7 - 12.5 | FIXED BROADCASTING BROADCASTING-SATELLITE 5.492 MOBILE except aeronautical mobile 5.487 5.487A | BROADCASTING-SATELLITE - ESIM downlink - HEST downlink SRD- Short Range Devices (Radiodeterm Applic. 30 MHz-12,4 GHz) | RR Appendix 30 RR Appendix 30A ERC DEC (00)08 ERC/DEC/(99)26 ECC/DEC/(06)03 ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 ECC/DEC(19)04 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 12.5 - 12.75 | FIXED-SATELLITE (space to earth) 5.484A 5.A15 (earth to space) | FIXED-SATELLITE (space to earth) - VSAT down link - SNG down link - ESV down link - HEST down link - AES down link - ESIM downlink | ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 ECC/DEC(19)04 ECC/DEC/(05)10 ECC/DEC/(06)03 ECC/DEC/(05)11 ERC/DEC/(99)26 |
| 12.75 - 13.25 | FIXED FIXED-SATELLITE (earth to space) 5.441, 5.A115 MOBILE Space Research (deep space) (space to earth) | FIXED FIXED-SATELLITE (earth to space) - SNG up link | CEPT T/R 12-02 ECC/DEC(19)04 |
| 13.25 - 13.4 | EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION 5.497 SPACE RESEARCH (active) 5.498A 5.499 | AERONAUTICAL RADIONAVIGATION |  |
| 13.4 - 13.65 | EARTH EXPLORATIONSATELLITE (active) FIXED-SATELLITE (space-to Earth) 5.499A 5.499B RADIOLOCATION SPACE RESEARCH 5.499C 5.499D Standard frequency and time signal-satellite (Earth-to-space) 5.499E 5.501B | RADIOLOCATION SRD- Short Range Devices (Radiodeterm. Devices 13.4-14 GHz) | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 13.65 – 13.75 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.501A Standard frequency and time signal-satellite (Earth-to-space) 5.501B | SRD- Short Range Devices (Radiodeterm. Devices 13.4-14 GHz) | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 13.75 – 14 | FIXED-SATELLITE (Earth-to-space) 5.484A RADIOLOCATION Earth exploration-satellite Standard frequency and time signal-satellite (Earth-to-space) Space research 5.502 5.503 | FIXED-SATELLITE (earth to space) - SNG up link RADIOLOCATION SRD- Short Range Devices (Radiodeterm. Devices 13.4-14 GHz) | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 14 – 14.25 | FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.504C 5.506A Space research 5.504A | FIXED-SATELLITE (earth to space) - VSAT up link - SNG up link - HEST up link 14.0-14.25 GHz - ESV up ink - AES uplink - ESIM uplink | ERC/REC 13-03 ECC/DEC/(03)04 ECC/DEC/(06)03 ECC/DEC/(05(10) ECC/DEC/(05)11 ECC/DEC/(17)04 ECC/DEC/(18)04 ECC/DEC/(18)05 |
| 14.25 - 14.3 | FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.508A Space research 5.504A | FIXED-SATELLITE (earth to space) - VSAT up link - SNG up link - ESV uplink - AES uplink - ESIM uplink | ERC/REC 13-03 ECC/DEC (03)04 ECC/DEC(05(10) ECC/DEC/(05)11 ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 |
| 14.3 - 14.4 | FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A | FIXED-SATELLITE (earth to space) - VSAT up link - SNG up link - ESIM uplink - ESV up ink - AES uplink | ERC/REC 13-03 ECC/DEC (03)04 ECC/DEC(05(10) ECC/DEC/(05)11 ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 |
| 14.4 - 14.47 | FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Space research (space-to-Earth) 5.504A | FIXED-SATELLITE (earth to space) - VSAT up link - SNG up link - ESIM uplink - ESV up link | ERC/REC 13-03 ECC/DEC (03)04 ECC/DEC(05(10) ECC/DEC/(05)11 ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 |
| 14.47 - 14.5 | FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.506 5.506B . MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radio astronomy 5.149 5.504A | FIXED-SATELLITE (earth to space) - VSAT up link - SNG up link - ESIM uplink - ESV - AES | ERC/REC 13-03 ECC/DEC (03)04 ECC/DEC(05(10) ECC/DEC/(05)11 ECC/DEC(17)04 ECC/DEC(18)04 ECC/DEC(18)05 |
| 14.5 - 14.75 | FIXED FIXED-SATELLITE (Earth-to-space) 5.509B 5.509C 5.509D 5.509E 5.509F 5.510 MOBILE Space research 5.509G | FIXED (14.5-14.62 paired with 15.23-15.35) | ERC/REC 12-07 |
| 14.75 – 14.8 | FIXED FIXED-SATELLITE (earth to space) 5.510 MOBILE Space Research 5.509G |  |  |
| 14.8 - 15.35 | FIXED MOBILE Space Research 5.A113 5.339 | FIXED (15.23-15.35 paired with 14.5-14.62) | ERC/REC 12-07 |
| 15.35 - 15.4 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 15.4 - 15.41 | AERONAUTICAL RADIONAVIGATION RADIOLOCATION 5.511E 5.511F |  |  |
| 15.41 - 15.43 | AERONAUTICAL RADIONAVIGATION RADIOLOCATION 5.511E 5.511F Aeronautical mobile (OR) 5.AA110 |  |  |
| 15.43 - 15.63 | FIXED-SATELLITE (earth to space) 5.511A AERONAUTICAL RADIONAVIGATION RADIOLOCATION 5.511E 5.511F 5.511C Aeronautical mobile (OR) 5.AA110 |  |  |
| 15.63 - 15.7 | RADIOLOCATION 5.511E 5.511F AERONAUTICAL RADIONAVIGATION Aeronautical mobile (OR) 5.AA110 |  |  |
| 15.7 - 16.6 | RADIOLOCATION |  |  |
| 16.6 - 17.1 | RADIOLOCATION Space Research (deep space)(earth to space) |  |  |
| 17.1 - 17.2 | RADIOLOCATION | SRD – Short Range Devices Radiodeterm. devices (17.1-17.3 GHz) | CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments |
| 17.2 - 17.3 | RADIOLOCATION EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) 5.513A | SRD – Short Range Devices Radiodeterm. devices (17.1-17.3 GHz) | CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments |
| 17.3 - 17.7 | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) 5.516A, 5.516B Radiolocation | FIXED-SATELLITE (space to earth) - ESOMP | ECC/DEC/(05)08 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 17.7 - 18.1 | FIXED FIXED-SATELLITE (space-to-Earth) 5.484A, 5.517A, 5.A116 (Earth-to-space) 5.516 MOBILE | FIXED FIXED-SATELLITE (space to earth) - ESOMP | CEPT T/R 12-03 ERC/DEC/(00)07 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 18.1 – 18.4 | FIXED FIXED-SATELLITE (space-to-Earth) 5.484A, 5.516B, 5.517A, 5.116 (Earth-to-space) 5.520 INTER-SATELLITE 5.A117 MOBILE 5.519 | FIXED FIXED-SATELLITE (space to earth) - ESOMP | CEPT T/R 12-03 ERC/DEC/(00)07 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 18.4 - 18.6 | FIXED FIXED-SATELLITE (space to earth) 5.484A, 5.516B, 5.517A, 5.A116 MOBILE | FIXED FIXED-SATELLITE (space to earth) - ESOMP | CEPT T/R 12-03 ERC/DEC/(00)07 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 18.6 - 18.8 | FIXED FIXED-SATELLITE (space to earth) 5.517A, 5.522B MOBILE except aeronautical mobile EARTH EXPLORATION-SATELLITE (passive) Space Research (passive) 5.522A | FIXED FIXED-SATELLITE (space to earth) - ESOMP | CEPT T/R 12-03 ERC/DEC/(00)07 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 18.8 - 19.3 | FIXED FIXED-SATELLITE (space to earth) 5.516B, 5.516B, 5.517A, 5.523A, 5.A116 INTER-SATELLITE 5.A117 MOBILE | FIXED FIXED-SATELLITE (space to earth) - ESOMP | CEPT T/R 12-03 ERC/DEC/(00)07 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 19.3 - 19.7 | FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) 5.517A, 5.523B, 5.523C, 5.523D, 5.523E INTER-SATELLITE 5.A117, 5.523X MOBILE | FIXED FIXED-SATELLITE (space to earth) - ESOMP | CEPT T/R 12-03 ERC/DEC/(00)07 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 19.7 - 20.1 | FIXED-SATELLITE (space-to-Earth) 5.484A, 5.484B, 5.516B, 5.527A, 5.A116 INTER-SATELLITE 5.A117 Mobile-satellite (space-to-Earth) | - HEST downlink FIXED-SATELLITE (space to earth) - ESOMP | ECC/DEC/(06)03 ECC/DEC/(05)08 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 20.1 - 20.2 | FIXED-SATELLITE (space-to-Earth) 5.484A, 5.484B, 5.516B, 5.527A ,5.A116 MOBILE-SATELLITE (space-to-Earth) 5.524 5.525 5.526 5.527 5.528 INTER-SATELLITE 5.A117 | - HEST down link FIXED-SATELLITE (space to earth) - ESOMP | ECC/DEC/(06)03 ECC/DEC/(05)08 ERC/DEC/(99)26 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 20.2 - 21.2 | FIXED-SATELLITE (space to earth) MOBILE-SATELLITE (space to earth) Standard Frequency and Time Signal-Satellite (space to earth) 5.B7(C)3 | FIXED-SATELLITE (space to earth) |  |
| 21.2 - 21.4 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) |  |  |
| 21.4 – 22 | FIXED MOBILE BROADCASTING-SATELLITE 5.208B 5.530A 5.530B | SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 22 - 22.21 | FIXED MOBILE except aeronautical mobile 5.B110, 5C110, 5.D110, 5.E110, 5F110 5.149 | FIXED SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | T/R 13-02 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 22.21 - 22.5 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) 5.149 5.532 | FIXED SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | T/R 13-02 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |  |
| 22.5 - 22.55 | FIXED MOBILE | FIXED SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | T/R 13-02 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 22.55 - 23.15 | FIXED MOBILE INTER-SATELLITE 5.538A SPACE RESEARCH (earth to space) 5.532A 5.149 | FIXED SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | T/R 13-02 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 23.15 – 23.55 | FIXED MOBILE INTER-SATELLITE 5.538A | FIXED SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | T/R 13-02 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 23.55 - 23.6 | FIXED MOBILE | FIXED SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | T/R 13-02 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 23.6 - 24 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 | SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) | ECC/DEC/(04)10 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 24 - 24.05 | AMATEUR AMATEUR-SATELLITE 5.150 | AMATEUR AMATEUR-SATELLITE SRD – Short Range Device (Automatic SRR 21.65- 26.65 GHz) (Non-specific SRD 24,0-24,25 GHz) ISM - Industrial, Scientific and Medical applications (24.0-24.25 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(04)10 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 24.05 - 24.25 | RADIOLOCATION Amateur Earth Exploration-Satellite (active) 5.150 | RADIOLOCATION Amateur SRD - Short Range Devices (Automatic SRR 21.65- 26.65 GHz) (Automotive radar 24,05-24,5 GHz) (Non-specific SRD 24,0-24,25 GHz) (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) ISM - Industrial, Scientific and Medical applications (24.0-24.25 GHz) | Amateur reg. CEPT T/R 60-01 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 ECC/DEC/(04)10 amendments Commison Dec 2005/50/EC Commison Dec 2017/2077/EC |
| 24.25 - 24.45 | FIXED MOBILE except aeronautical mobile 5.338A 5.532AB | MOBILE SRD - Short Range Devices (Automatic SRR 21.65- 26.65 GHz) (Automotive radar 24,05-24,5 GHz) (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) | ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 CEPT Report 068 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 Commison Dec 2005/50/EC Commison Dec Commison Dec 2017/2077/EC ECC/DEC(11)02 |
| 24.45 - 24.65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile 5.338A 5.532AB | MOBILE SRD - Short Range Devices (Automatic SRR 21.65- 26.65 GHz) (Automotive radar 24,05-24,5 GHz) (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) | ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 CEPT Report 068 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 Commison Dec 2005/50/EC Commison Dec Commison Dec 2017/2077/EC ECC/DEC(11)02 |
| 24.65 – 24.75 | FIXED FIXED-SATELLITE (Earth-to-space) 5.532B INTER-SATELLITE MOBILE except aeronautical mobile 5.338A 5.532AB | MOBILE SRD - Short Range Devices (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) (Automatic SRR 21.65- 26.65 GHz) | ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 CEPT Report 068 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 Commison Dec 2005/50/EC Commison Dec 2017/2077/EC ECC/DEC(11)02 |
| 24.75 - 25.25 | FIXED FIXED-SATELLITE (Earth-to-space) 5.532B MOBILE except aeronautical mobile 5.338A 5.532AB | MOBILE SRD - Short Range Devices (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) (Automatic SRR 21.65- 26.65 GHz) | ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 CEPT Report 068 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 Commison Dec 2005/50/EC Commison Dec 2017/2077/EC ECC/DEC(11)02 |
| 25.25 - 25.5 | FIXED 5.534A INTER-SATELLITE 5.536 MOBILE 5.338A 5.532AB Standard frequency and time signal-satellite (Earth-to-space) | MOBILE SRD - Short Range Devices (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) (Automatic SRR 21.65- 26.65 GHz) | ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 CEPT Report 068 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 Commison Dec 2005/50/EC Commison Dec 2017/2077/EC ECC/DEC(11)02 |
| 25.5 - 27 | EARTH EXPLORATION-SATELLITE (space-to-Earth) 5.536B FIXED 5.534A INTER-SATELLITE 5.536 MOBILE 5.338A 5.532AB SPACE RESEARCH (space-to-Earth) 5.536C Standard frequency and time signal-satellite (Earth-to-space) 5.536A | EARTH EXPLORATION-SATELLITE (space-to-Earth) 5.536B MOBILE SPACE RESEARCH (space-to-Earth) 5.536C SRD - Short Range Devices (Tank level prob. radar (24,05-27 GHz) (Indstrial level prob radar 24,05-27 GHz) (Automatic SRR 21.65- 26.65 GHz) | ECC/REC/(19)01 ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 CEPT Report 068 CEPT T/R 70—03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 Commison Dec 2005/50/EC Commison Dec 2017/2077/EC ECC/DEC(11)02 |
| 27 - 27.5 | FIXED INTER-SATELLITE 5.536 MOBILE 5.338A 5.532AB | MOBILE | CEPT Report 068 ECC/DEC/(18)06 Commission Dec 2019/784 Commission Dec 2020/590 |
| 27.5 - 28.5 | FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B, 5.517A, 5.539, 5.A116 INTER-SATELLITE 5.A117 MOBILE 5.538 5.540 | FIXED (27,8285-28,4445 GHz paired with 28,9485-29,4525 GHz FIXED SATELLITE (earth to space) - Fixed Earth station (coord. with FS) - ESOMP (27.5-27,8285, 28,4445-28,9485, 29,4525-29,5 GHz) | CEPT T/R 13-02 Annex C ECC/DEC(05)01 ECC/DEC(13)01 ECC/DEC/(15)04 |
| 28.5 - 29.1 | FIXED FIXED-SATELLITE (Earth-to-space) 5.484A, 5.516B, 5.517A, 5.523A, 5.539, 5.A116 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 INTER-SATELLITE 5.A117 5.540 | FIXED (27,8285-28,4445 GHz paired with 28,9485-29,4525 GHz FIXED SATELLITE (earth to space) - Fixed Earth station (coord. with FS) - ESOMP (27.5-27,8285, 28,4445-28,9485, 29,4525-29,5 GHz) | CEPT T/R 13-02 Annex C ECC/DEC(05)01 ECC/DEC(13)01 ECC/DEC/(15)04 |
| 29.1 - 29.5 | FIXED FIXED-SATELLITE (earth to space) 5.523C, 5.523E, 5.535A, 5.539, 5.541A MOBILE INTER-SATELLITE 5.A117 Earth Exploring-Satellite (earth to space) 5.541 5.540 | FIXED (27,8285-28,4445 GHz paired with 28,9485-29,4525 GHz FIXED SATELLITE (earth to space) - Fixed Earth station (coord. with FS) - ESOMP (27.5-27,8285, 28,4445-28,9485, 29,4525-29,5 GHz) | CEPT T/R 13-02 Annex C ECC/DEC(05)01 ECC/DEC(13)01 |
| 29.5 - 29.9 | FIXED-SATELLITE (Earth-to-space) 5.484A, 5.484B, 5.516B, 5.527A, 5.539, 5.A116 INTER-SATELLITE 5.A117 Mobile-satellite (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 5.540 | FIXED-SATELLITE (earth to space) - ESOMP - HEST up link | ECC/DEC/(06)03 ECC/DEC/(05)08 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 29.9 - 30 | FIXED-SATELLITE (earth to space) 5.539 5.484A, 5.516B, 5.5X, 5.A15, 5.A116 MOBILE-SATELLITE (earth to space) INTER-SATELLITE 5.A117 Earth Exploring-Satellite (earth to space) 5.541 5.543 5.525, 5.526, 5.527, 5.543, 5.538, 5.540 | FIXED-SATELLITE (earth to space) - ESOMP - HEST up link | ECC/DEC/(06)03 ECC/DEC/(05)08 ECC/DEC/(13)01 ECC/DEC/(15)04 |
| 30 - 31 | FIXED-SATELLITE (Earth-to-space) 5.338A MOBILE-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (space-to-Earth) 5.B7(C)3 |  | ECC/DEC/10(02) |
| 31 - 31.3 | FIXED 5.338A 5.543B MOBILE Standard frequency and time signal-satellite (space-to-Earth) Space research 5.544 5.545 5.149 | FIXED | ECC/REC/(02)02 |
| 31.3 - 31.5 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  | ECC/DEC/10(02) |
| 31.5 - 31.8 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) Fixed Mobile except aeronautical mobile 5.149 |  |  |
| 31.8 - 32 | RADIONAVIGATION FIXED S5.547A SPACE RESEARCH (deep sp.) (sp. to earth) 5.547, 5.548 | FIXED | ERC/REC(01)02 |
| 32 - 32.3 | RADIONAVIGATION FIXED 5.547A SPACE RESEARCH (deep sp.) (sp. to earth) 5.547, 5.548 | FIXED | ERC/REC(01)02 |
| 32.3 - 33 | RADIONAVIGATION FIXED 5.547A INTER-SATELLITE 5.547, 5.548 | FIXED | ERC/REC(01)02 |
| 33 - 33.4 | RADIONAVIGATION FIXED 5.547A 5.547 | FIXED | ERC/REC(01)02 |
| 33.4 - 34.2 | RADIOLOCATION |  |  |
| 34.2 - 34.7 | RADIOLOCATION SPACE RESEARCH (deep sp.) (earth to sp.) |  |  |
| 34.7 - 35.2 | RADIOLOCATION Space Research |  |  |
| 35.2 - 35.5 | METEOROLOGICAL AIDS RADIOLOCATION |  |  |
| 35.5 - 36 | EARTH EXPLORATION-SATELLITE (active) METEOROLOGICAL AIDS RADIOLOCATION SPACE RESEARCH (active) |  |  |
| 36 - 37 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) 5.149 |  |  |
| 37 - 37.5 | FIXED MOBILE 5.550B SPACE RESEARCH (space to earth) 5.547 | FIXED (37-38.178 paired with 38.318-39.5) | CEPT T/R 12-01 Low and medium capacity links |
| 37.5 - 38 | FIXED FIXED-SATELLITE (space-to-Earth) 5.550C, 5. A91D MOBILE except aeronautical mobile 5.550B SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 | FIXED (37-38.178 paired with 38.318-39.5) | CEPT T/R 12-01 Low and medium capacity links ERC/DEC/(00)02 |
| 38 - 39.5 | FIXED 5.550D FIXED-SATELLITE (space-to-Earth) 5.550C MOBILE 5.550B Earth exploration-satellite (space-to-Earth) 5.547 | FIXED (37-38.178 paired with 38.318-39.5) | CEPT T/R 12-01 Low and medium capacity links ERC/DEC/(00)02 |
| 39.5 - 40 | FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.550C MOBILE 5.550B MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 5.550E |  | ERC/DEC/(00)02 ECC/REC/(22)02 |
| 40 - 40.5 | EARTH EXPLORATION-SATELLITE (Earth-to-space) FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.550C MOBILE 5.550B MOBILE-SATELLITE (space-to-Earth) SPACE RESEARCH (Earth-to-space) Earth exploration-satellite (space-to-Earth) 5.550E |  | ERC/DEC/(00)02 ECC/REC/(22)02 |
| 40.5 - 41 | FIXED FIXED-SATELLITE (space-to-Earth) 5.550C LAND MOBILE 5.550B BROADCASTING BROADCASTING-SATELLITE Aeronautical mobile Maritime mobile 5.547 | MOBILE | ECC/DEC/(23)01 ECC/DEC/(22)06 ECC/REC/(22)02 ECC/REC/(22)01 Commission Dec 2024/1983 |
| 41 – 42.5 | FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.550C LAND MOBILE 5.550B BROADCASTING BROADCASTING-SATELLITE Aeronautical mobile Maritime mobile 5.547 5.551F 5.551H 5.551I | MOBILE | ECC/DEC/(23)01 ECC/DEC/(22)06 ECC/REC/(22)02 ECC/REC/(22)01 Commission Dec 2024/1983 |
| 42.5 - 43.5 | FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE except aeronautical mobile 5.550B RADIO ASTRONOMY 5.149 5.547 | MOBILE | ECC/DEC/(22)06 ECC/REC/(22)02 ECC/REC/(22)01 ECC/DEC/(23)01 Commission Dec 2024/1983 |
| 43.5 - 47 | MOBILE 5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.554 |  |  |
| 47 - 47.2 | AMATEUR AMATEUR-SATELLITE | AMATEUR AMATEUR-SATELLITE | Amateur reg. CEPT T/R 60-01 |
| 47.2 – 47.5 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.552A | FIXED FIXED-SATELLITE (Earth-to-space) | ECC/DEC/(21)01 |
| 47.5 – 47.9 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 (space-to-Earth) 5.516B 5.554A MOBILE | FIXED FIXED-SATELLITE (Earth-to-space) | ECC/DEC/(05)08 ECC/DEC/(21)01 |
| 47.9 – 48.2 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.552A | FIXED FIXED-SATELLITE (Earth-to-space) | ECC/DEC/(21)01 |
| 48.2 – 48.54 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 (space-to-Earth) 5.516B 5.554A 5.555B MOBILE | FIXED FIXED-SATELLITE (Earth-to-space) | ERC/REC 12-11 ECC/DEC/(05)08 ECC/DEC/(21)01 |
| 48.54 – 49.44 | FIXED FIXED-SATELLITE (Earth-to-space) 5.550C 5.552 MOBILE 5.149 5.340 5.555 | FIXED FIXED-SATELLITE (Earth-to-space) | ERC/REC 12-11 ECC/DEC/(21)01 |
| 49.44 – 50.2 | FIXED FIXED-SATELLITE (Earth-to-space) 5.338A 5.550C 5.552 (space-to-Earth) 5.516B 5.554A 5.555B MOBILE | FIXED FIXED-SATELLITE (Earth-to-space) | ERC/REC 12-11 ECC/DEC/(05)08 ECC/DEC/(21)01 |
| 50.2 - 50.4 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) 5.340 |  |  |
| 50.4 - 51.4 | FIXED FIXED-SATELLITE (Earth-to-space) 5.338A 5.550C MOBILE Mobile-satellite (Earth-to-space) | FIXED FIXED-SATELLITE (Earth-to-space) | ERC/REC 12-11 ECC/DEC/(21)01 |
| 51.4 - 52.4 | FIXED FIXED-SATELLITE (Earth-to-space) 5.555C MOBILE 5.338A 5.547 5.556 | FIXED FIXED-SATELLITE (Earth-to-space) | ERC/REC 12-11 ECC/DEC/(21)01 |
| 52.4 – 52.6 | FIXED 5.338A MOBILE 5.547 5.556 | FIXED | ERC/REC 12-11 |
| 52.6 - 54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) 5.340, 5.556 |  |  |
| 54.25 - 55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE 5.556A SPACE RESEARCH (passive) |  |  |
| 55.78 – 56.9 | EARTH EXPLORATION-SATELLITE (passive) FIXED 5.557A INTER-SATELLITE 5.556A MOBILE 5.558 SPACE RESEARCH (passive) 5.547 5.557 |  |  |
| 56.9 - 57 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE 5.558A MOBILE 5.558 SPACE RESEARCH (passive) 5.547 5.557 |  |  |
| 57 – 58.2 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE 5.556A MOBILE 5.558 SPACE RESEARCH (passive) 5.547 5.557 | FIXED SRD – Short Range Devices (Non-specific SRD 57-64 GHz) (Industrial level prob radar 57-64 GHz) (Tank level prob radar 57-64 GHz) (Wideband data systems 57-71 GHz | ECC/DEC(11)02 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 58.2 - 59 | EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) 5.556, 5.547 | FIXED SRD – Short Range Devices (Non-specific SRD 57-64 GHz) (Industrial level prob radar 57-64 GHz) (Tank level prob radar 57-64 GHz) (Wideband data systems 57-71 GHz | ECC/DEC(11)02 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 59 - 59.3 | EARTH EXPLORATION-SATELLITE (passive) FIXED INTER-SATELLITE 5.556A MOBILE 5.558 SPACE RESEARCH (passive) RADIOLOCATION 5.559 5.138 | SRD – Short Range Devices (Non-specific SRD 57-64 GHz) (Industrial level prob radar 57-64 GHz) (Tank level prob radar 57-64 GHz) (Wideband data systems 57-71 GHz | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 59.3 - 64 | FIXED INTER-SATELLITE MOBILE 5.558 RADIOLOCATION 5.559 5.138 | SRD – Short Range Devices (Non-specific SRD 57-64 GHz) (Industrial level prob radar 57-64 GHz) (Tank level prob radar 57-64 GHz) (Wideband data systems 57-71 GHz) (Transp & Telematics Dev 63-64 GHz) (ITS 63.72-65,88 GHz) | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)10 2005/50/EC ECC/DEC/(09)01 ECC/DEC(11)02 |
| 64 - 65 | FIXED INTER-SATELLITE MOBILE except aeronautical mobile 5.556, 5.547 | FIXED SRD- Short Range Devices (Wideband data systems 57-71 GHz) (ITS 63.72-65,88 GHz) | ECC/REC/(05)02 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(09)01 |
| 65 - 66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH 5.547 | FIXED SRD- Short Range Devices (Wideband data systems 57-71 GHz) (ITS 63.72-65,88 GHz) | ECC/REC/(05)02 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(09)01 |
| 66 - 71 | INTER-SATELLITE MOBILE 5.553 5.558 5.559AA MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.554 | SRD- Short Range Devices (Wideband data systems 57-71 GHz) (Security scanners indoor 69,8-80,5 GHz) | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments |
| 71 - 74 | FIXED FIXED-SATELLITE (space to earth) MOBILE MOBILE-SATELLITE (space to earth) | FIXED SRD- Short Range Devices (Security scanners indoor 69,8-80,5 GHz) | ECC/REC/(05)07 CEPT T/R 70-03 |
| 74 - 76 | FIXED FIXED-SATELLITE (space to earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space Research (space to earth) 5.561 | FIXED SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) (Security scanners indoor 69,8-80,5 GHz) | ECC/REC/(05)07 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 76 – 77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-Satellite Space Research (space to earth) 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) (Transport&Traffic Telematics 76-77 GHz) (Obstacle detection radars 76-77 GHz) (Automotive SRR 77-81 GHz) (Security scanners indoor 69,8-80,5 GHz) | Amateur reg. CEPT T/R 60-01 ERC/REC 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(04)03 Commison Dec 2004/545/EC ECC/DEC/(11)02 ECC/DEC/(16)01 ECC/DEC/(21)02 |
| 77.5 - 78 | AMATEUR AMATEUR-SATELLITE RADIOLOCATION 5.559B Radio astronomy Space research (space-to-Earth) 5.149 | AMATEUR AMATEUR-SATELLITE SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) (Automotive SRR 77-81 GHz) (Security scanners indoor 69,8-80,5 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(04)03 Commison Dec 2004/545/EC CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 78 - 79 | RADIOLOCATION Amateur Amateur-Satellite Radio Astronomy Space Research (space to earth) 5.149, 5.560 | Amateur Amateur-Satellite SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) (Automotive SRR 77-81 GHz) (Security scanners indoor 69,8-80,5 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(04)03 Commison Dec 2004/545/EC CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 79 – 81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-Satellite Space Research (space to earth) 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) (Automotive SRR 77-81 GHz) (Security scanners indoor 69,8-80,5 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(04)03 Commison Dec 2004/545/EC CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 81 - 84 | FIXED 5.338A FIXED-SATELLITE (earth to space) MOBILE MOBILE-SATELLITE (earth to space) RADIO ASTRONOMY Space Research (space to earth) 5.149, 5.561A | FIXED SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) | ECC/REC/(05)07 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 84 - 86 | FIXED 5.338A FIXED-SATELLITE (earth to space) 5.561B MOBILE RADIO ASTRONOMY 5.149 | FIXED SRD – Short Range Devices (Tank Level Probing Radar 75-85 GHz) (Industrial Level Prob Radar 75-85 GHz) | ECC/REC/(05)07 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC(11)02 |
| 86 - 92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 92 - 94 | FIXED 5.338A MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |  |
| 94 - 94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio Astronomy 5.562, 5.562A |  |  |
| 94.1 - 95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 |  |  |
| 95 - 100 | FIXED MOBILE RADIONAVIGATION RADIONAVIGATION-SATELLITE RADIO ASTRONOMY RADIOLOCATION 5.149, 5.554 |  |  |
| 100 - 102 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340, 5.341 |  |  |
| 102 - 105 | FIXED MOBILE RADIO ASTRONOMY 5.149, 5.341 |  |  |
| 105 –109.5 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) 5.562B 5.149, 5.341 |  |  |
| 109.5 – 111.8 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340, 5.341 |  |  |
| 111.8 – 114.25 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) 5.562B 5.149, 5.341 |  |  |
| 114,25 – 116 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340, 5.341 |  |  |
| 116 – 119,98 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE 5.562C SPACE RESEARCH (passive) 5.341 | SRD – Short Range Devices (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 119,98 – 122.25 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE S5.562C SPACE RESEARCH (passive) 5.138, 5.341 | SRD – Short Range Devices (Non-specific devices 122-123 GHz) (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(22)03 Commison Dec 2026/883 |
| 122.25 – 123 | FIXED INTER-SATELLITE MOBILE 5.558 Amateur 5.138 | Amateur SRD – Short Range Devices (Non-specific devices 122-123 GHz) (Gen indoor surv. Radar, EVR, IVR -122,25-130 GHz) (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | Amateur reg. CEPT T/R 60-01 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(22)03 Commison Dec 2026/883 |
| 123 – 130 | FIXED-SATELLITE (SPACE TO EARTH) MOBILE-SATELLITE (SPACE TO EARTH) RADIONAVIGATION RADIONAVIGATION-SATELLITE Radio Astronomy 5.562D 5.149, 5.554 | SRD – Short Range Devices (Gen indoor surv. radar, EVR, IVR -122,25-130 GHz) (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 130 – 134 | FIXED INTER-SATELLITE MOBILE 5.558 EARTH EXPLORATION-SATELLITE (active) 5.562E RADIO ASTRONOMY 5.149, 5.562A | SRD – Short Range Devices (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 134 – 136 | AMATEUR AMATEUR-SATELLITE Radio Astronomy | AMATEUR AMATEUR-SATELLITE SRD – Short Range Devices (Gen indoor surv. radar, EVR, IVR -134-148,5 GHz) (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 136 – 141 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-Satellite 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (Gen indoor surv. radar, EVR, IVR -134-148,5 GHz) (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 141 – 148.5 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 | SRD – Short Range Devices (Gen indoor surv. radar, EVR, IVR -134-148,5 GHz) (LPR, TLPR, CDR - 116-148,5 GHz) (RDI-S 116-148,5 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 148.5 – 151.5 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 151.5 – 155.5 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION 5.149 | (RDI-S 151,5-164 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 155.5 – 158.5 | FIXED MOBILE RADIO ASTRONOMY 5.149 | (RDI-S 151,5-164 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 158.5 – 164 | FIXED FIXED-SATELLITE (space to earth) MOBILE MOBILE-SATELLITE (space to earth) | (RDI-S 151,5-164 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 164 – 167 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 167 – 174.5 | FIXED FIXED-SATELLITE (space to earth) MOBILE S5.558 INTER-SATELLITE 5.149, 5.562D | SRD – Short Range Devices (LPR, TLPR, CDR – 167-182 GHz) (RDI-S 167-182 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 174.5 – 174.8 | FIXED MOBILE 5.558 INTER-SATELLITE | SRD – Short Range Devices (LPR, TLPR, CDR – 167-182 GHz) (RDI-S 167-182 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 174.8 – 182 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE 5.562H SPACE RESEARCH (passive) | SRD – Short Range Devices (RDI – 174,8-182 GHz) (LPR, TLPR, CDR – 167-182 GHz) (RDI-S 167-182 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 182 - 185 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 185 - 190 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE 5.562H SPACE RESEARCH (passive) | SRD – Short Range Devices (RDI – 185-190 GHz) (RDI-S 185-190 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 190 – 191.8 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) 5.340 |  |  |
| 191.8 - 200 | FIXED INTER-SATELLITE MOBILE 5.558 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.149, 5.341 5.554 | (RDI-S 191.8 – 200 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 200 - 209 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340, 5.341, 5.563A |  |  |
| 209 – 217 | FIXED FIXED-SATELLITE (earth to space) MOBILE RADIO ASTRONOMY 5.149, 5.341 | (RDI-S 209 – 226 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 217 - 226 | FIXED FIXED-SATELLITE (earth to space) MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) 5.562B 5.149, 5.341 | (RDI-S 209 – 226 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 226 – 231.5 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |
| 231.5 - 232 | FIXED MOBILE Radiolocation | SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 232 – 235 | FIXED FIXED-SATELLITE (space to earth) MOBILE Radiolocation | SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 235 – 238 | EARTH EXPLORATION-SATELLITE (passive) 5.B114 FIXED FIXED-SATELLITE (space to earth) MOBILE SPACE RESEARCH (passive) 5.563A, 5.563B | SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 238 – 239,2 | FIXED FIXED-SATELLITE (space to earth) MOBILE RADIONAVIGATION RADIONAVIGATION-SATELLITE RADIOLOCATION | SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 239,2 – 240 | EARTH EXPLORATION-SATELLITE (passive) FIXED-SATELLITE (space to earth) RADIONAVIGATION RADIONAVIGATION-SATELLITE RADIOLOCATION | SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 240 – 241 | EARTH EXPLORATION-SATELLITE (passive) RADIOLOCATION | SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 241 – 242,2 | EARTH EXPLORATION-SATELLITE (passive RADIOLOCATION RADIOASTRONOMY Amateur Amateur-Satellite 5.138, 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (Non-specific devices 244-246 GHz) (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | Amateur reg. CEPT T/R 60-01 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(22)03 Commison Dec 2026/883 |
| 242,2 – 244,2 | RADIOLOCATION RADIOASTRONOMY Amateur Amateur-Satellite 5.138, 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (Non-specific devices 244-246 GHz) (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | Amateur reg. CEPT T/R 60-01 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(22)03 Commison Dec 2026/883 |
| 244,2 -247,2 | EARTH EXPLORATION-SATELLITE (passive RADIOLOCATION RADIOASTRONOMY Amateur Amateur-Satellite 5.138, 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (Non-specific devices 244-246 GHz) (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | Amateur reg. CEPT T/R 60-01 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(22)03 Commison Dec 2026/883 |
| 247,2 - 248 | RADIOLOCATION RADIOASTRONOMY Amateur Amateur-Satellite 5.138, 5.149 | Amateur Amateur-Satellite SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | Amateur reg. CEPT T/R 60-01 CEPT T/R 70-03 Commison Dec 2006/771/EC + amendments ECC/DEC/(22)03 Commison Dec 2026/883 |
| 248 - 250 | AMATEUR AMATEUR-SATELLITE Radio Astronomy 5.149 | AMATEUR AMATEUR-SATELLITE SRD – Short Range Devices (RDI – 231,5-250 GHz) (LPR, TLPR, CDR – 231,5-250 GHz) (RDI-S 231,5-250 GHz) | Amateur reg. CEPT T/R 60-01 ECC/DEC/(22)03 CEPT T/R 70-03 Commison Dec 2026/883 |
| 250 - 252 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) RADIO ASTRONOMY 5.340, 5.563A |  |  |
| 252 - 265 | FIXED MOBILE MOBILE-SATELLITE (earth to space) RADIO ASTRONOMY RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.149, 5.554 | (RDI-S 252-260 GHz) | ECC/DEC/(22)03 Commison Dec 2026/883 |
| 265 - 275 | FIXED FIXED-SATELLITE (earth to space) MOBILE RADIO ASTRONOMY 5.149, 5.563A |  |  |
| 275 - 3000 | (not allocated) 5.564A 5.565 |  |  |

In addition following EU and ECC Decisions have been implemented in Iceland:

- EU Decision 2024/1467/EC on the harmonisation of radios pectrum for equipment using UWB technology in the Union

- ECC/DEC/(06)04 on harmonised use, exemption from individual licensing and free circulation of devices using UWB technology in bands below 10,6 GHz

- ECC/DEC/(07)01 on harmonised use, exemption from individual licensing and free circulation of of Material Sensing Devices using UWB technology.